# Appendix D: Sample Emergency Response Checklists

# Example only

1.	Helicopter Crash on Deck (No Fire)

| Helicopter Crash on Deck (No Fire) |  |  |  |
| --- | --- | --- | --- |
| 1 | Respond to the incident |  |  |
| All | 1.1 | Flying debris/crash hazards | Seek protection |
| HLO | 1.2 | Raise the alarm | Sound Alarm Inform Radio Operator |
|  | 1.3 | Check for signs of fire | If Fire starts see card ‘Helicopter Crash on Deck (With Fire)’ |
| WARNING |  |  |  |
| Do not approach helicopter until blades and rotors have stopped moving |  |  |  |
|  | 1.5 | Plan approach | Identify safe pathways |
|  | 1.6 | Cockpit/Cabin | Check for signs of movement |
| 2 | Evacuate persons from the aircraft |  |  |
| HLO | 2.1 | Assist persons exiting aircraft | HDA1; HDA2 as required |
|  | 2.2 | Evacuees (Uninjured/minor injuries) | Direct or assist to safe area (HDAs i.a.w. ERP) |
| 3 | Conduct Rescue |  |  |
| HLO | 3.1 | Select Rescue equipment | As appropriate |
| HDA | 3.2 | Release trapped crew/passengers | Execute |
|  | 3.3 | Rescued crew/passengers | Assist to safe area (i.a.w. ERP) |
| 4 | Provide assistance to evacuees |  |  |
| Med | 4.1 | Assess injuries | Seek assistance (Installation Medical personnel as req) |
| HLO | 4.2 | Move injured personnel/evacuees | As directed by medical officer |
| 5 | Conclude incident activities |  |  |
| HLO | 5.1 | Contain fuel/oil spillages (See card xx Fuel Spill) | Deploy spill response kit Foam blanket (as required) |
|  | 5.2 | Assess damage to helideck/installation | Alert HMI as required |
| CAUTION |  |  |  |
| The crash site must be preserved for purposes of Accident Investigation |  |  |  |
| HLO | 5.3 | Debris on Helideck | Remove (if posing risk to safety) |
|  | 5.4 | Preserve evidence | Secure helideck/effected areas |
|  | 5.5 | Examine aircraft for damage caused during rescue | Record for Accident Investigation |

| 6 | Complete incident debrief |  |  |
| --- | --- | --- | --- |
| HLO | 6.1 | Complete Incident Report |  |
| ALL | 6.2 | Participate in Accident investigation | Attend debrief/interview |
| HLO | 6.3 | Identify problems/lessons learned | Record faults/defects/drill change requirements |

2.	Helicopter Crash on Deck with Fire

| Helicopter Crash on Deck with Fire |  |  |  |
| --- | --- | --- | --- |
| 1 | Respond to the incident |  |  |
| ALL | 1.1 | Helicopter crash on deck | Seek protection |
| HLO | 1.2 | Warn Helideck Team Warn Installation | Helideck loop Sound Alarm |
|  | 1.3 | Check for signs of fire | Visual |
| HLO | 1.3.1 | With no signs of fire | Conduct rescue Monitor for signs of fire (See Card ‘Heli Crash on Deck No Fire’) |
| HLO | 1.3.2 | With signs of fire | Sound Alarm Alert Helideck Team Alert OIM |
| 2 | Fight Fire |  |  |
| HLO/HDA | 2.1 | DIFFS (if available) | Activate (if not automatic) |
| HLO | 2.2 | Select upwind Monitor | Check wind |
| Fire Team | 2.3 | FMS (if available) | Start pumps Lay foam blanket ‘Knock down’ fire |
|  | 2.4 | Hoses (if required) | Run out Connect inductors/branches |
|  | 2.5 | Make foam/water (as required) | Lay foam blanket/spray |
| 3 | Approach aircraft |  |  |
| HLO | 3.1 | Rescue Team | Brief |
|  | 3.2 | FMS | Stop operation |
| Fire Team | 3.3 | Hoses | Pulse spray Cool fuselage |
| HLO | 3.4 | Assess risk | Fire/explosion |
|  | 3.4.1 | If risk acceptable | Send in rescue team |
|  | 3.4.2 | If risk not acceptable | Continue to fight fire |
| 4 | Rescue occupants |  |  |
| Fire Team | 4.1 | Fuselage | Check integrity |
| WARNING |  |  |  |
| Beware ‘flash-over’. Ventilate with positive pressure using non-aspirated branch. |  |  |  |
| Rescue Team | 4.2 | Conduct search | Cockpit/Cabin |
|  | 4.3 | Release crew/pax | Evacuate to safety |
|  | 4.4 | Brief HLO | SITREP |
| HLO | 4.5 | Brief OIM | Full SITREP |
| Med Assist | 4.6 | Assess injuries | Request Medical assistance |
| HLO | 4.7 | Other Agencies | Co-ordinate |